Unit Standard Topic: Construct a Furniture Carcase: Bedside Cabinet (Year 12 Furniture)

Week 1
What a “Carcase” Is (and What a Bedside Cabinet Needs)

- Carcase (casework): The main “box” structure of a cabinet—typically two sides, a top, a bottom, and a back—that creates a rigid, square container.

- Bedside cabinet requirements:

  - Square and rigid so doors/drawers fit and operate smoothly.

  - Durable joints that resist racking (twisting out of square).

  - Clean edges and accurate reveals for a professional look.

- Common parts (typical build): side panels, top, bottom, fixed shelf (optional), back panel, face frame or edge lipping (optional), and a door/drawer unit (optional).

- Key terms:

  - Racking: diagonal distortion of the box under force.

  - Reveal: the consistent gap around a door/drawer front.

  - Carcase square: diagonals are equal and edges are 90°.

Week 2
Material Selection, Grain Direction, and Planning for Movement

- Material options:

  - Plywood: stable, strong, ideal for student casework; edges often need edgebanding/lipping.

  - Solid wood: strong and attractive, but moves seasonally; best used with correct grain planning.

  - MDF: flat and good for paint, but heavy and weaker at screws/edges unless reinforced.

- Grain direction: keep grain running vertically on sides and front-to-back on top/bottom for appearance and strength.

- Wood movement: solid wood expands/contracts across the grain; use joinery and back attachment methods that don’t cause splitting.

- Planning steps:

  - Final dimensions and cut list

  - Joint selection (strength + appearance + available tools)

  - Back panel choice (thin panel in groove, or rebated back)

  - Hardware plan (hinges, runners, pulls) before cutting.

Week 3
Accuracy Fundamentals: Marking Out, Cutting List, and “Reference Faces”

- Why marking out matters: Most carcase problems come from small layout errors that multiply across parts.

- Reference face / reference edge: A chosen “true” face and edge on each component used for all measurements so everything stays consistent.

- Marking tools (and their purpose):

  - Tape/steel rule: overall dimensions.

  - Try square/combination square: 90° layout and checking.

  - Marking gauge: parallel lines (shoulders, rebates, housings).

  - Knife vs pencil: knife lines increase accuracy for joinery shoulders.

- Best practice process:

  - Prepare stock (flat, straight, squared)

  - Mark from reference faces

  - Label parts clearly (inside/outside, top/bottom orientation)

  - Dry-fit mindset: measure, cut, check, adjust before glue.

Week 4
Dowel Joint as a Widening Joint (Panel Joining for Sides/Top/Bottom)

- What it is: Dowels align and strengthen edge-to-edge joints when making wider panels from narrower boards.

- Where used in a bedside cabinet: creating solid-wood side panels or a top from multiple boards.

- Strength and limitations:

  - Adds alignment and some strength, but the primary glue surface is the edge grain.

  - Requires accurate drilling so boards don’t “step” at the glue line.

- Key steps:

  - Joint edges straight and square; test by closing the seam dry.

  - Mark dowel positions consistently from the same reference edge.

  - Drill to depth with a stop; dry-fit for flushness.

  - Glue evenly; clamp with cauls to keep faces flush.

- Quality checks: tight seam, flush faces, no twist after clamping, minimal squeeze-out for easier cleanup.

Week 5
Box Pin Joint (Finger Joint) for Corners: Strength + Visual Detail

- What it is: Interlocking “fingers” at a corner; large glue surface and strong mechanical lock.

- Best use on a bedside cabinet: visible front corners for a premium look (especially on solid wood).

- Advantages: excellent resistance to pulling apart and racking; decorative when exposed.

- Considerations:

  - Requires consistent spacing and precise cutting.

  - Works best when stock thickness is consistent and square.

- Common methods:

  - Table saw with jig, router with jig, or careful hand-tool layout and cutting.

- Quality checks: fingers should seat fully without crushing; corner should pull square with light clamping; end grain should align cleanly.

Week 6
Through Housing Joint (Dado) for Fixed Shelves and Internal Strength

- What it is: A slot across the full width of a board (often a side panel) to receive another piece (like a shelf).

- Where used: fixed shelf, bottom, or top support—adds stiffness and helps keep the carcase square.

- Why it matters: a fixed shelf set into housings dramatically increases resistance to racking.

- Cutting approaches:

  - Router with straight bit + guide

  - Table saw with repeated passes

  - Hand tools (saw + chisel) for controlled fitting

- Fit standard: “snug, not forced.” Too tight can split plywood veneers or cause assembly stress; too loose reduces strength and accuracy.

- Pro tip: Use the shelf piece as a story stick/template to set the housing width precisely.

Week 7
Rebate Butt Joint and Mitre Joint (Carcase Edges and Clean Lines)

- Rebate butt joint: One piece has a rebate (rabbet) so the mating piece sits into a ledge.

  - Common uses: attaching a back panel, setting in the top/bottom, or forming cleaner corner alignment than a plain butt joint.

  - Benefits: increased glue area, helps alignment, hides some end grain depending on orientation.

- Mitre joint: Two pieces meet at 45° to form a 90° corner, often used for a clean, continuous grain look.

  - Common uses: decorative corners, frames, or when you want to hide end grain.

  - Limitations: weak if only glued end grain; often reinforced with splines, biscuits, or keys.

- Decision factors: strength needs, tool access, visibility of joinery, time, and repeatability for student builds.

Week 8
Tools and Equipment: Safe, Correct Use in Carcase Construction

- Measuring/marking: tape, steel rule, try square, marking gauge, knife, pencil, bevel gauge (for mitres).

- Cutting and shaping:

  - Hand: panel saw/tenon saw, chisels, block plane, shoulder plane, mallet.

  - Machines (if available): table saw/track saw, mitre saw, router, drill press, random orbit sander.

- Boring and joinery: brad-point bits, dowel jig, clamps, router bits (straight, rebate), dado setup (where allowed).

- Clamping and assembly: sash clamps/bar clamps, corner squares, cauls, mallet + scrap block.

- Safety essentials (teacher-ready reminders):

  - Eye protection, hearing protection, dust control.

  - Clamp work securely before drilling/routing.

  - Keep hands clear of cutting path; use push sticks and guards.

  - Check tool setup (bit tight, fence locked, depth set) before powering on.

Week 9
Assembly Workflow: From Dry Fit to Glue-Up (Keeping It Square)

- Dry fit first: assemble without glue to confirm joinery fit, orientation, and clamp strategy.

- Glue-up plan: stage clamps, blocks, square, and a damp cloth before applying glue.

- Squareness checks:

  - Measure diagonals: equal diagonals indicate square.

  - Use a large square at corners; check twist on a flat surface.

- Clamping strategy:

  - Clamp just enough to close joints; excessive pressure can bow panels.

  - Use cauls to keep edges flush and distribute pressure.

- Back panel role: a properly fitted back (rebated or housed) locks the carcase square and increases rigidity.

Week 10
Finish Quality: Troubleshooting, Evaluation Criteria, and Extension Ideas

- Common issues + causes:

  - Out of square: uneven clamping, inaccurate cuts, rushing glue-up.

  - Gaps at joints: layout errors, dull tools, poor test fits.

  - Tear-out (especially plywood): incorrect blade/bit choice, cutting against grain, no scoring.

- Evaluation criteria (student rubric ideas):

  - Accuracy: dimensions, squareness, consistent reveals.

  - Joinery: fit, alignment, glue cleanliness, strength.

  - Surface prep: sanding sequence, sharp edges eased, consistent finish.

  - Function: door/drawer operation, stability, no racking.

- Extensions: add a drawer with runners, install soft-close hinges, add adjustable shelf pins, apply edge lipping, or reinforce mitres with splines for advanced practice.