Unit Standard Topic: Power Tools and Maintenance (Year 12)

- Focus: Safe selection, operation, maintenance, and troubleshooting of common portable power tools

- Core outcomes: Identify tools and functions; operate safely; choose the right tool; maintain; troubleshoot; apply electrical safety; evaluate performance

- Essential mindset: “Safety + setup + maintenance = accuracy and longevity”

Sub Topic 1
Common Portable Power Tools: Identification & Functions

- Cordless drill/driver: Drilling holes and driving fasteners; key variables are speed, torque, and clutch settings (to prevent stripping).

- Impact driver: High-torque driving for long screws/lag fasteners; uses rotational impacts to reduce cam-out.

- Circular saw: Straight cuts in sheet goods and framing lumber; blade size and tooth count affect cut quality and speed.

- Jigsaw: Curved cuts and interior cutouts; best for thinner stock and shapes; depends heavily on blade type.

- Random-orbit sander: Surface smoothing and finish prep; balances stock removal and swirl reduction with proper grit progression.

- Angle grinder (if used): Cutting/grinding metal or masonry; requires correct wheel and guard position.

- Key concept: Tool performance is primarily driven by bit/blade selection, setup, and feed rate—not just power.

Sub Topic 2
Tool Selection: Matching Tool, Material, and Task

- Start with the task: cut, drill, fasten, grind, sand, or shape.

- Match the material:

  - Wood: tooth geometry and blade type matter (rip vs. crosscut; carbide tips).

  - Metal: use rated cutting wheels/blades; control heat; deburr edges.

  - Plastics/composites: reduce melting with correct tooth count and slower feed.

- Precision vs. speed: jigsaw for curves; circular saw for straight, fast cuts; sanding for finish quality.

- Project constraints: access, clearance, noise/dust, power source (corded vs. cordless), and required accuracy.

- Rule of thumb: Choose the tool that achieves the cut/fastener with the least risk and least rework.

Sub Topic 3
Pre-Use Inspection & Setup (Before You Pull the Trigger)

- Visual inspection: cracks, missing screws, damaged guards, loose handles, bent base plates/shoes.

- Power check:

  - Corded: inspect insulation, plug, strain relief; confirm switch returns to “off.”

  - Cordless: check battery casing, terminals, and secure latch; avoid overheated packs.

- Consumables: confirm blade/bit is sharp, correct, and rated for tool RPM.

- Work area: stable stance, clear floor, good lighting, no trip hazards, dust control planned.

- Workpiece control: clamp whenever possible; confirm cut path avoids cords and bench edges.

- Functional check: quick run-up to listen for abnormal vibration or sound (then stop and diagnose).

Sub Topic 4
Safe Operation: Handling, Body Position, and Control

- Two-hand control when designed for it (e.g., circular saw, angle grinder).

- Stance and line of fire: keep your body out of the kickback path; stand slightly offset.

- Feed rate: let the tool cut—forcing increases heat, binds blades, and raises kickback risk.

- Guard discipline: never defeat guards; confirm they move freely and return properly.

- Trigger control: start at full control, allow tool to reach speed before contact, and wait for full stop before setting down.

- Cords and hoses: route behind you; maintain slack without draping across the cut line.

- Stop-work criteria: unusual noise, burning smell, wobble, excessive sparking, or repeated tripping—shut down and inspect.

Sub Topic 5
Electrical Safety for Powered Tools

- Grounding: use 3-prong grounded tools/outlets unless tool is double-insulated (marked accordingly).

- GFCI protection: use GFCI in garages, outdoors, and any potentially damp area.

- Cord handling: never carry tools by the cord; avoid sharp edges and heat sources; don’t yank plugs.

- Water hazards: keep tools and cords away from standing water; never operate in wet conditions unless rated.

- Extension cords: use the correct gauge for load and length (undersized cords cause voltage drop and overheating).

- Lockout awareness: disconnect power/remove battery before changing blades/bits, clearing jams, or adjusting guards.

Sub Topic 6
Routine Maintenance: Cleaning, Lubrication, and Wear Checks

- Cleaning: blow out vents carefully (low pressure), wipe housings, and remove packed dust from guards/shoes.

- Lubrication (only as specified): some tools require periodic lubrication of gears or pivot points—follow manufacturer guidance.

- Inspect wear parts:

  - Brushes (brushed motors): check length and spring tension; replace as needed.

  - Bearings: listen for grinding; feel for shaft play.

  - Switches and triggers: ensure smooth operation; no sticking.

- Battery care: avoid storing fully depleted; keep contacts clean; don’t leave packs in extreme heat/cold.

- Recordkeeping: tag tools with service dates and issues; consistent logs prevent repeat failures.

Sub Topic 7
Replacing Consumables & Worn Parts (Correctly and Safely)

- Blades/bits/discs: match arbor size, RPM rating, and application; verify direction arrows.

- Sander pads and abrasives: replace worn hook-and-loop pads; use proper grit progression (coarse → fine).

- Belts (belt sanders): align tracking and confirm tension; re-check after brief run time.

- Guards and base plates: replace if cracked, bent, or not square—these affect both safety and accuracy.

- Post-replacement test: run tool unloaded for 10–15 seconds; check vibration, noise, and guard function before returning to work.

Sub Topic 8
Troubleshooting Common Issues (Symptoms → Likely Causes → Fix)

- Tool won’t start: dead battery, faulty cord, tripped breaker, bad switch/brushes → verify power source, inspect cord, check brushes.

- Overheating/burning smell: dull blade, forced feed, blocked vents, undersized extension cord → replace consumable, clean vents, correct cord.

- Excessive vibration: bent blade/disc, loose arbor, worn bearings → replace and re-seat consumable, tighten, service bearings.

- Poor cut quality (burning/tear-out): wrong blade, wrong speed, poor support/clamping → change blade, adjust technique, support workpiece.

- Repeated kickback/binding: blade pinching, misaligned fence/shoe, warped stock → improve support, confirm alignment, adjust cut plan.

- Safety rule: if the fix requires opening electrical housings beyond routine brush access, follow school/shop policy for qualified service.

Sub Topic 9
Evaluating Tool Performance: When to Maintain vs. Repair vs. Replace

- Efficiency indicators: slower cut speed, reduced torque, battery runtime drop, frequent stalls, inconsistent speed under load.

- Accuracy indicators: drifting cuts, out-of-square base/shoe, excessive runout in chucks/arbors.

- Cost-benefit check: compare repair cost + downtime vs. replacement cost + reliability gains.

- Safety threshold: any tool with compromised guards, electrical damage, or recurring faults should be removed from service immediately.

- Recommendation format (students):

  - Observed issue → probable cause → evidence → action (maintain/repair/replace) → safety notes

Sub Topic 10
Skill Demonstration & Performance Checklist (Student-Facing)

- Identify & describe: correctly name at least 6 tools and explain primary use + one limitation each.

- Select appropriate tool: choose tool + blade/bit + settings for a given material and task; justify choice.

- Demonstrate safe operation: PPE as required, correct stance/hand placement, guarded operation, controlled start/stop, safe set-down.

- Perform basic maintenance: clean vents, inspect cord/battery, check guard movement, inspect/replace consumable, document service.

- Troubleshoot scenario: diagnose from symptoms and propose a safe corrective action.

- Exit criteria: tool is returned clean, unplugged/battery removed, stored properly, and any issues are reported/tagged.