Unit Standard Topic: Use of Portable Sanding Machines (Year 11)

Sub Topic 1
What a Portable Sander Does (and Why It Matters)

- Definition: A portable sanding machine is a handheld power tool that abrades (wears down) a surface using sandpaper to shape material or improve finish quality.

- Common shop goals: remove mill marks, level joints, refine edges, prepare surfaces for paint/stain/clear coat, and blend repairs.

- Key idea: Sanding is controlled material removal—your choices (tool, grit, pressure, and direction) determine speed, flatness, and final appearance.

- Quality factors: flatness (no waves/dishes), scratch pattern (consistent and appropriate), edge control (no rounding unless intended), and cleanliness (dust affects finish adhesion).

Sub Topic 2
Types of Portable Sanders (Match Tool to the Job)

- Random Orbital Sander (ROS): Round pad with random motion; best general-purpose for smooth finishes with minimal swirl marks.

- Finishing/Orbital (Sheet) Sander: Square pad using sanding sheets; good for light finishing and getting into corners better than ROS (but slower removal).

- Belt Sander: High material removal; used for flattening, leveling, and rapid stock removal; easiest to gouge if misused.

- Detail/Mouse Sander: Small triangular pad; for tight areas, inside corners, and small repairs.

- Disc/Angle Sander (with sanding disc): Aggressive; often used on metal or heavy removal; requires strong control and PPE.

- Selection rule: Choose the least aggressive tool that can do the job efficiently while protecting the workpiece.

Sub Topic 3
Main Parts & Functions (Identify Components)

- Motor: Provides power; overload can cause overheating and reduced life.

- Switch/Trigger + Lock (if present): Turns tool on/off; lock supports continuous run—use cautiously.

- Speed Control (variable speed models): Matches sanding speed to material and finish needs.

- Pad/Platen: The surface that holds sandpaper; must be flat and in good condition for even results.

- Backing Pad (ROS): Cushion layer that affects cut and finish; worn pads cause uneven sanding and vibration.

- Dust Collection (bag/canister/port): Captures dust to improve visibility, finish quality, and respiratory safety.

- Grip/Handle & Housing: Control and stability; proper grip reduces fatigue and improves accuracy.

- Tracking Knob (belt sanders): Centers the belt to prevent edge damage and belt loss.

Sub Topic 4
Sandpaper Types & Abrasives (Choose the Right Paper)

- Grit number: Lower grit = larger particles = faster removal; higher grit = finer particles = smoother finish.

- Abrasive materials:

  - Aluminum Oxide: Durable; common for wood and general use.

  - Silicon Carbide: Sharp; good for finishes, plastics, and some metals; often used for wet/dry sanding.

  - Ceramic: Very durable, fast-cutting; good for heavy removal and tough materials.

- Backing types: paper (standard), cloth (strong for belts), film (uniform for fine finishing).

- Coating:

  - Open-coat: More space between grains; reduces clogging on softwoods/paint.

  - Closed-coat: More abrasive per area; faster cut but clogs easier.

- Hole patterns (ROS/sheets): Must match pad holes to maintain dust extraction and reduce loading.

Sub Topic 5
Selecting Grit for Material & Desired Finish (Practical Progressions)

- General principle: Start only as coarse as needed, then progress in steps to remove previous scratches.

- Typical wood progression (finish-ready):

- 80→120→150→180→220

- 80→120→150→180→220 (adjust for species and finish type).

- Paint/finish removal: Start

- 60–80

- 60–80 (with caution), then refine.

- Between coats (light scuff sanding):

- 220–320

- 220–320 (or per product instructions).

- Metal (sanding vs. polishing): Often

- 80–120

- 80–120 for shaping, then

- 180–320+

- 180–320+ for refining before polishing steps.

- Rule of thumb: Don’t jump too far (e.g.,

- 80→220

- 80→220); it wastes time because deep scratches remain.

- Checkpoints: After each grit, inspect under raking light and feel with fingertips—scratches should be uniform before moving on.

Sub Topic 6
Safe Operation & PPE (Non-Negotiables)

- Eye protection: Safety glasses (face shield as needed).

- Respiratory protection: Dust mask/respirator appropriate to material (especially MDF, finishes, or metal dust).

- Hearing protection: Many sanders exceed safe dB levels during extended use.

- Workholding: Clamp/secure the workpiece; never “chase” a moving board with a running sander.

- Pre-use checks: Cord/battery condition, pad condition, paper installed correctly, dust collection attached/emptied, no loose clothing.

- Start/stop discipline: Start the tool off the surface (especially belt/angle sanders), then contact gently; lift before shutting off to avoid gouges.

- Dust hazard awareness: Fine dust can be hazardous and can accumulate—use extraction and good housekeeping; treat unknown coatings as potentially hazardous.

Sub Topic 7
Effective Sanding Techniques (Control = Quality)

- Let the abrasive cut: Use light-to-moderate pressure; excess pressure causes swirl marks, heat, and uneven surfaces.

- Keep the pad flat: Tilting rounds edges and creates “dishes.”

- Movement patterns:

  - ROS: slow, overlapping passes; maintain a steady pace (too fast reduces cutting).

  - Belt sander: move with the grain when possible; keep moving to avoid divots; use two hands.

- Edge and corner strategy: Ease up near edges; consider hand-sanding or a softer pad to avoid burn-through/rounding.

- Scratch management: Sand with the grain for final passes on wood; remove cross-grain scratches before finishing.

- Heat control: Heat indicates too much pressure, dull paper, clogged paper, or speed too high—adjust before damage occurs.

Sub Topic 8
Basic Maintenance (Keep the Tool Performing)

- Clean after use: Blow off/brush vents (follow shop rules), wipe housing, and empty dust bag/canister.

- Inspect consumables:

  - Replace worn pads/backing pads (loss of hook-and-loop grip = uneven sanding).

  - Replace belts/discs/sheets when dull or loaded (clogged abrasive burns and scratches).

- Check fasteners and alignment: Especially on belt sanders (tracking and belt condition).

- Cord/battery care: No sharp bends or pinches; store batteries properly; inspect contacts.

- Dust collection system: Ensure ports aren’t clogged; confirm hole alignment; use correct adapter to maintain airflow.

- Preventive mindset: A well-maintained sander produces better finishes, runs cooler, and is safer to control.

Sub Topic 9
Troubleshooting Common Issues (Cause → Fix)

- Uneven sanding / waves:

  - Causes: rocking the tool, worn pad, too much pressure, poor workholding.

  - Fixes: lighten pressure, keep pad flat, replace pad, secure workpiece.

- Swirl marks (ROS):

  - Causes: skipping grits, dirty paper, too much pressure, moving too fast, poor dust extraction.

  - Fixes: improve dust collection, slow down, reduce pressure, step back one grit and re-sand evenly.

- Clogged paper (loading):

  - Causes: softwood/resin, finish dust, too fine a grit too soon, no extraction.

  - Fixes: switch to open-coat, improve extraction, change paper more often.

- Motor overheating:

  - Causes: blocked vents, pressing too hard, dull paper, extended continuous use.

  - Fixes: clean vents, reduce pressure, replace abrasive, allow cool-down cycles.

- Excess vibration:

  - Causes: damaged pad, off-center disc, worn bearings (advanced), debris on pad.

  - Fixes: re-seat paper, clean/replace pad; tag tool out if mechanical issue persists.

Sub Topic 10
Evaluate Work Quality (Inspection + Adjustments)

- Visual inspection: Use raking light (light held low to the surface) to reveal scratches, dents, and uneven spots.

- Tactile inspection: Run fingertips lightly—feel for ridges, pits, glue squeeze-out, and edge rounding.

- Scratch pattern test: The surface should show a consistent scratch pattern from the final grit, with no deeper scratches.

- Flatness check: Use a straightedge for critical surfaces (tabletops, joints).

- Finish readiness: Remove dust thoroughly (vacuum + tack cloth if appropriate); dust left behind can cause finish defects.

- Adjustment decisions:

  - If deep scratches remain: drop back a grit and re-sand uniformly.

  - If edges are rounding: reduce pressure, change technique, or switch tools for edge control.

- Efficiency habit: Change paper early; sharp abrasive saves time and improves results.