Unit Standard Topic: Using Fixing Hardware in Furniture Making (Year 12)

Sub Topic 1
What “Fixing Hardware” Is and Why It Matters
Fixing hardware is the set of mechanical fasteners and connectors used to join wood and other furniture materials. The right hardware choice affects strength, appearance, serviceability, and safety.

- Key idea: Different joints face different forces—shear (sideways sliding), tension (pulling apart), and racking (twisting). Hardware should match the forces.

- Selection factors: material thickness, load/weight, indoor vs. outdoor use (corrosion risk), visibility (hidden vs. exposed), and whether the piece must be disassembled.

- Quality check: choose hardware with consistent sizing, proper plating/coating, and no stripped threads or bent parts.

Sub Topic 2
Basic Fixing Hardware: Categories + Choosing the Right One
Common categories you’ll use in furniture making:

- Fasteners: nails and screws (primary structural attachment).

- Movement hardware: hinges (allow doors/lids to swing).

- Retention hardware: catches and locks (keep doors/drawers closed, provide security).

- Demountable connectors: knock-down fittings (flat-pack/ready-to-assemble furniture).
How to choose:

- Strength vs. speed: nails are fast; screws give better holding power and removability.

- Hidden vs. visible: choose head styles and finishes to match design (brass, black, zinc).

- Wood behavior: account for wood movement (seasonal expansion/contraction) and avoid over-restricting panels.

Sub Topic 3
Nails: Types, Uses, and Best Practices
Nails are driven fasteners that rely on friction and wood compression. They are quick for assembly but less removable and can loosen under vibration.

- Common types in furniture/casework: brad nails, finish nails, panel pins, upholstery tacks (specialty).

- Where nails work well: attaching thin back panels, decorative trim, temporary positioning before gluing, light-duty components.

- Best practices:

  - Pre-mark layout to avoid edge splits; keep nails away from very thin edges.

  - Drive straight; set heads slightly below the surface for filling (finish work).

  - Combine with glue for stronger assemblies when appropriate.

- Avoid: using nails alone for heavily loaded joints (chairs, structural rails) where screws/joinery are safer.

Sub Topic 4
Screws: Anatomy, Types, and Holding Power
Screws are threaded fasteners that create strong mechanical grip and can be removed for repairs.

- Anatomy: head (drives), shank, threads, point.

- Common types: wood screws, chipboard/MDF screws, cabinet screws, self-tapping (for some metals), pocket-hole screws.

- Head styles:

  - Countersunk/flat: sits flush—good for furniture surfaces.

  - Pan/washer head: higher bearing surface—useful in hardware mounting.

- Holding power basics: longer embedment and proper pilot holes improve strength; the screw should bite mainly in the “receiving” piece, not bind both pieces.

Sub Topic 5
Screws: Pilot Holes, Clearance Holes, and Avoiding Splits
Accurate drilling is what makes screws reliable and clean-looking.

- Pilot hole: drilled into the receiving piece so threads cut without splitting.

- Clearance hole: drilled through the first piece so the screw can pull parts together tightly.

- Countersink: creates a conical recess so a flat-head screw sits flush.

- Practical tips:

  - Drill pilots near edges and in hardwoods every time.

  - Use a stop collar or tape on the bit to control depth.

  - Drive straight and stop when snug—overdriving strips the hole, especially in MDF/particleboard.

- Troubleshooting: if threads strip, use a larger screw, a repair plug/dowel, or a threaded insert (for high-wear connections).

Sub Topic 6
Butt Hinges: Parts, Installation, and Alignment
A butt hinge is a two-leaf hinge with a pin that allows a door to swing; it’s common in traditional furniture and cabinets.

- Parts: two leaves, knuckle, pin (and sometimes removable pin).

- Sizing basics: hinge size relates to door height/weight; heavier doors need larger or more hinges.

- Installation steps (high-level):

  - Mark hinge locations consistently (often near top and bottom).

  - Create a recess/mortise so the leaf sits flush with the wood surface.

  - Pre-drill screw holes to prevent splitting and misalignment.

  - Test swing and adjust gaps (reveal) for even spacing.

- Common mistakes: misaligned mortises causing binding, using screws too short, skipping pilot holes, and uneven door gaps.

Sub Topic 7
Catches and Locks: Function, Types, and Placement
Catches and locks control closure and, in the case of locks, add security.

- Catches (keep closed): magnetic catches, roller catches, ball catches, spring catches.

- Locks (secure): cam locks, cabinet locks, hasp-style solutions (context-dependent).

- Choosing the right option:

  - User experience: easy open/close vs. firm retention.

  - Door movement: match the catch to the hinge type and door alignment.

  - Material and finish: corrosion resistance and visual match to other hardware.

- Placement tips: install where force is balanced (often near the opening edge). Ensure the strike plate/receiver aligns precisely to avoid rattling or failure.

Sub Topic 8
Knock-Down (KD) Fittings: Flat-Pack Joinery and When to Use It
Knock-down fittings are connectors designed for furniture that needs to be assembled/disassembled (RTA/flat-pack).

- Common KD systems: cam lock + dowel, connector bolts, barrel nuts/cross dowels, confirmat screws (panel furniture).

- Advantages: repeatable assembly, easier transport, replaceable parts, consistent manufacturing.

- Trade-offs: visible hardware in some designs, reliance on precise drilling, and potential loosening if over-tightened or assembled repeatedly.

- Best uses: wardrobes, desk frames, cabinet carcasses, shelving units—especially with sheet goods (MDF/particleboard/plywood).

- Key requirement: accuracy—hole spacing and depth must match the fitting system exactly.

Sub Topic 9
Hardware Planning: Layout, Materials, and Corrosion/Finish Choices
Good hardware use starts before cutting and drilling.

- Layout discipline: measure twice, mark clearly, use a story stick or template for repeated hole/hinge locations.

- Material compatibility:

  - Hardwood vs. softwood affects pilot sizing.

  - MDF/particleboard often needs specialty screws or KD systems to avoid stripping.

- Corrosion resistance:

  - Indoor: zinc-plated is often sufficient.

  - Humid/outdoor: consider stainless steel or coated hardware to reduce rust.

- Finish coordination: match hinge/catch/lock finishes (brass, nickel, black) to design intent; consider visibility and whether you’ll conceal with caps/plugs.

Sub Topic 10
Workshop Safety + Quality Checks (Ready-to-Use Checklist)
Safety essentials:

- Wear eye protection when drilling/driving fasteners; keep hands clear of the drive line.

- Clamp workpieces before drilling; avoid holding small parts by hand.

- Use correct bit types (Phillips, Pozidriv, Torx) to prevent cam-out and injuries.

Quality check before calling it “done”:

- Are all screw heads seated correctly (flush/countersunk as intended)?

- Do doors swing smoothly with even gaps and no binding at hinges?

- Do catches engage reliably without slamming or excessive force?

- Are KD joints tight and square (no wobble/racking)?

- Is the hardware finish consistent and free from scratches?