Unit Standard Topic: Constructing Working Drawings for Furniture Making (Year 13)

Week 1
What a “Working Drawing” Is (and Why It Matters in Furniture Making)

- Working drawing (definition): A complete set of technical drawings that communicates exactly how a furniture piece will be made—shape, size, materials, joinery, and assembly details—so the builder can produce it accurately and consistently.

- Purpose in a shop setting:

  - Prevents errors and wasted material by clarifying measurements and construction steps before cutting.

  - Ensures parts fit together (especially critical for joinery like mortise-and-tenon or dadoes).

  - Supports teamwork: different people can fabricate parts and still match the same specifications.

- Typical components of a working drawing set: orthographic views, dimensions, notes, materials list, joinery details, and sometimes an exploded/assembly view.

Week 2
Understanding Job Requirements (Reading the Brief Like a Builder)

- Job requirements (definition): The constraints and expectations for the furniture piece—function, size, style, materials, timeline, and quality standards.

- Key questions to clarify before drawing:

  - Function & load: What will the piece hold/support? Expected weight and daily use?

  - Space & fit: Where will it go? Required overall dimensions and clearances (doors, drawers, legroom).

  - Style & aesthetics: Modern, traditional, rustic? Visible joinery or hidden? Finish expectations?

  - Materials: Solid wood vs. plywood vs. MDF; species; thickness availability; grain direction requirements.

  - Hardware: Drawer slides, hinges, pulls—brand/model affects hole placement and clearances.

  - Budget & time: Can the joinery and finish level be achieved within constraints?

- Deliverables to confirm: drawing format, required views, dimensioning standard, and any teacher/shop rubric.

Week 3
From Requirements to Specifications (Turning Needs into Measurable Decisions)

- Specification (definition): A measurable, testable design choice (e.g., “top thickness 19 mm,” “drawer clearance 2 mm per side”).

- Common furniture specs to decide early:

  - Overall dimensions: height, width, depth; ergonomic targets (seat height, table height).

  - Material thicknesses: match stock sizes to reduce milling and cost.

  - Joinery type & location: where strength is needed vs. where speed is needed.

  - Tolerances/allowances: space for wood movement, hardware clearance, and finishing thickness.

  - Safety and durability: rounded edges, stable base, anti-rack features (stretchers, panels).

- Design reality check: Sketch a quick parts breakdown (carcass, top, legs, rails, panels) before committing to final drawings.

Week 4
Drawing Equipment: Core Tools and What Each One Does

- Drawing board or drafting surface: Provides a flat, stable base for accurate lines.

- T-square/straightedge: Creates horizontal lines and aligns other tools.

- Set squares/triangles (e.g., 45° and 30°/60°): Produces accurate verticals and common angles.

- Scale rule (architect/metric scale): Lets you draw accurately at reduced/enlarged scale.

- Pencils/lead grades (concept): Harder lead for light construction lines; softer for darker final lines.

- Eraser & erasing shield: Corrects errors cleanly without damaging nearby lines.

- Compass/dividers: Draw circles/arcs and transfer measurements.

- Templates (circles, ellipses, lettering): Speeds up consistent shapes and text.

- Drafting tape/clips: Holds paper steady to prevent shifting.

Week 5
Drawing Equipment: Accuracy, Care, and Setup (Professional Habits)

- Accuracy practices:

  - Keep tools clean; dust and chips can shift rulers and change measurements.

  - Use consistent line weight: light construction lines first, darker object lines later.

  - Check calibration: ensure triangles are not chipped; verify right angles.

- Paper and layout:

  - Choose appropriate sheet size for the object and required views.

  - Leave margins for title block, notes, and revision info.

- Digital option (shop-relevant): CAD can improve precision and revision speed, but drawings must still follow correct standards (views, dimensions, notes).

- Shop connection: Accurate drawings reduce “measure-twice/cut-once” mistakes by making the measuring phase more reliable.

Week 6
Orthographic Projection (The Language of Technical Furniture Drawings)

- Orthographic projection (definition): A method of representing a 3D object using multiple 2D views—each view shows true size and shape from a specific direction.

- Why furniture makers use it: It communicates exact part geometry without perspective distortion.

- Primary views (most common in furniture):

  - Front elevation: what you see looking straight at the front.

  - Top/plan view: what you see looking down (critical for layout and overhangs).

  - Side elevation: reveals depth, leg profiles, panel offsets, and joinery alignment.

- Key rule: Each view must align logically—edges and features should project consistently between views.

Week 7
Applying Orthographic Projection to Furniture Parts and Joinery

- Showing joinery clearly:

  - Use hidden lines to show features not visible (e.g., dado depth) when needed.

  - Use section views (cut-through drawings) to reveal internal joints (tenons, grooves, rebates/rabbets).

  - Use detail views (enlarged area) for small or complex joints and hardware holes.

- Furniture-specific features to represent well:

  - Grain direction (often via notes) to prevent weak parts and to control movement/appearance.

  - Panel construction and allowance for expansion (floating panels, elongated holes).

  - Edge treatments (round-over, chamfer) with labeled sizes.

- Consistency: If a hole pattern is shown in one view, it must match in all related views.

Week 8
Drawing Requirements: What Must Be Included in a Working Drawing Set

- Title block (definition): The information panel identifying the drawing—project name, scale, date, drafter, and revision number.

- Required drawing content (typical expectations):

  - Correct number of views to fully define the piece (no missing geometry).

  - Clear dimensions: overall sizes plus critical part sizes and locations.

  - Notes: materials, finish, hardware specs, and special instructions.

  - Part identification: labels for components (e.g., “Leg A,” “Side Rail B”).

  - Scale: stated clearly; avoid mixing scales without labeling each view.

- Line conventions: object lines, hidden lines, center lines, and section hatching should be used consistently so the drawing is readable to others.

Week 9
Dimensioning and Drawing Requirements (Furniture-Making Precision)

- Dimensioning (definition): Adding measurements to a drawing so the object can be built without guessing.

- Best practices for dimensioning furniture:

  - Dimension to functional surfaces (e.g., inside width for drawer fit, not just outside width).

  - Avoid duplicating dimensions that can conflict after revisions.

  - Include hole sizes, depths, and locations for hardware (hinges, slides, knock-down fittings).

  - Indicate thickness of panels/boards; many errors come from assumed stock thickness.

  - Include joinery dimensions: tenon length, mortise depth, dado width/depth, shoulder locations.

- Tolerances/allowances to note:

  - Clearance for moving parts (drawers/doors).

  - Space for seasonal wood movement (especially across grain).

  - Finish thickness considerations in tight fits.

Week 10
Workflow: Constructing a Working Drawing for Furniture (Start-to-Finish Checklist)

- Step 1: Interpret the job brief → list constraints, hardware, materials, and success criteria.

- Step 2: Concept sketch + rough dimensions → confirm proportions and usability.

- Step 3: Parts breakdown (cut list draft) → identify every component and its role.

- Step 4: Create orthographic views → front/top/side first; add sections/details next.

- Step 5: Add dimensions and notes → prioritize critical fits and joinery.

- Step 6: Review for buildability → check alignment between views, missing dimensions, and assembly order.

- Step 7: Finalize drawing set → title block, revision control, readability, and print/export readiness.

- Quality check prompts (student-friendly):

  - Could a different person build this with no extra instructions?

  - Are all parts measurable without “math in the shop”?

  - Do the drawings show how it comes apart/assembles and where hardware goes?