Unit Standard Topic: Demonstrate Knowledge of Hardware used in Furniture Making and its Application (Year 13)

Week 1
What “Furniture Hardware” Means (and Why It Matters)

- Furniture hardware: Manufactured components added to furniture to make it function, assemble, move, adjust, or look finished.

- Hardware choices affect:

  - Strength & safety (load capacity, secure joints)

  - User experience (smooth sliding, soft-close, adjustability)

  - Durability (wear, corrosion resistance, service life)

  - Appearance (style, finish, visible vs. concealed)

- Common decision factors: material type, intended use, environment (humidity, outdoor exposure), maintenance, and cost.

Week 2
Decorative Hardware: Purpose and Common Types

- Decorative hardware: Hardware that is intentionally visible and contributes to the furniture’s style and identity.

- Typical examples:

  - Knobs and pulls (drawers/doors): change the look and improve grip

  - Backplates/escutcheons: protect surfaces and add detail around pulls or keyholes

  - Corner guards & trim pieces: protect edges and add a finished look

  - Nailheads, tacks, and decorative screws: accent upholstery or wood surfaces

- Key terms:

  - Finish (e.g., brushed, polished, matte): surface appearance + wear resistance

  - Patina: natural or designed aging effect on metal surfaces

  - Fastener head style: influences both appearance and tool choice.

Week 3
Decorative Hardware: Application Skills (Layout, Fastening, Quality)

- Layout and alignment:

  - Use templates/jigs for repeatable placement of knobs/pulls.

  - Maintain consistent centerlines, spacing, and symmetry on multi-drawer projects.

- Fastening methods:

  - Machine screws + threaded inserts (strong, serviceable, ideal for repeated use)

  - Wood screws (quick, common; choose correct length to avoid blow-through)

  - Bolts with washers/nuts (high strength where access allows)

- Quality checks:

  - Hardware sits flush (no rocking), pulls feel solid, no stripped holes.

  - No finish damage: prevent scratches by using painter’s tape, soft jaws, and correct drivers.

Week 4
Furniture Fittings: What They Are and Where They’re Used

- Furniture fittings: Often functional components—many are concealed—that enable movement, connection, or adjustment.

- Major categories:

  - Hinges: butt hinges, concealed (European) hinges, pivot hinges

  - Drawer slides: side-mount, under-mount, center-mount; soft-close options

  - Connector systems: confirmat screws, cam-and-dowel connectors, knock-down fittings

  - Shelf support systems: shelf pins, standards, hidden supports

- Why fittings matter: they determine fit, alignment, and how the piece performs over time (sagging, racking, loosening).

Week 5
Furniture Fittings: Selecting the Right Option (Performance Criteria)

- Key selection criteria:

  - Load rating: expected weight + safety margin (books vs. clothing vs. tools)

  - Material compatibility: hardwood, plywood, particleboard require different fasteners/anchors

  - Motion and feel: smoothness, soft-close, self-close, opening angle, over-travel

  - Installation tolerance: some fittings require very accurate boring/drilling patterns

  - Serviceability: ease of adjustment, replacement, and tightening over time

- Common installation requirements:

  - Accurate hole spacing and depth control for concealed hinges and slides

  - Correct reveal and gap between doors/drawers for consistent alignment.

Week 6
Table Hardware: Core Components and Their Applications

- Table hardware supports structure, stability, and sometimes expandability.

- Common table hardware:

  - Corner braces/angle brackets: reinforce apron-to-leg connections

  - Figure-8 fasteners / tabletop clips: attach top to base while allowing wood movement

  - Threaded inserts + bolts: strong, removable leg attachment (flat-pack friendly)

  - Table extension slides: enable expanding leaves; require precise alignment

  - Leaf locks/alignment pins: keep leaves flush and prevent shifting

- Big concept: wood movement (seasonal expansion/contraction).

  - Tabletop attachment hardware must allow movement to prevent splitting or warping.

Week 7
Base Fittings: Leveling, Glides, Casters, and Floor Protection

- Base fittings protect floors, improve stability, and change how furniture moves.

- Common base fittings:

  - Glides/felt pads: reduce scratching; best for light furniture on smooth floors

  - Nylon/plastic glides: durable, lower friction; good for tile/laminate

  - Leveling feet: compensate for uneven floors; common in cabinets, workbenches, commercial furniture

  - Casters: add mobility; choose locking vs. non-locking; consider wheel material (rubber vs. polyurethane)

- Selection criteria:

  - Floor type (hardwood, carpet, concrete)

  - Weight capacity (total load ÷ number of supports, with safety margin)

  - Stability needs (locking casters, wider stance, anti-tip considerations).

Week 8
Installation Workflow: From Planning to Final Checks

- Step-by-step process used in professional shops:

  - Define function (soft-close drawer? removable legs? adjustable shelf?)

  - Choose hardware that matches load, material, and style

  - Read the spec sheet (hole spacing, screw type, clearances)

  - Mock-up/test fit on scrap when possible

  - Mark and drill with depth stops and jigs; verify square and parallel

  - Install and adjust (hinge adjustments, slide alignment, leveling feet)

  - Quality control: smooth motion, even gaps, secure fasteners, no rubbing

- Common causes of failure: incorrect screw length, misaligned pilot holes, ignoring wood movement, overtightening into engineered boards.

Week 9
Safety, Tools, and Best Practices with Furniture Hardware

- Safety priorities:

  - Eye protection during drilling and driving fasteners

  - Secure workpieces with clamps to prevent spinning or shifting

  - Use correct driver bits to prevent cam-out and injuries

- Best practices:

  - Pilot holes sized to material prevent splitting and stripped screws

  - Use threaded inserts for repeated assembly/disassembly

  - Match metal types and finishes to reduce corrosion (especially in humid spaces)

  - Keep hardware organized and labeled (part numbers, left/right slides, screw lengths)

- Professional standard: hardware should be consistent, aligned, smooth, and serviceable.

Week 10
Demonstrating Mastery (Student Evidence + Mini Performance Tasks)

- Evidence of knowledge (what “proficient” looks like):

  - Correctly identify decorative hardware, fittings, table hardware, and base fittings

  - Explain why each is chosen (load, movement, aesthetics, adjustability)

  - Install hardware accurately and troubleshoot alignment issues

- Mini performance tasks (choose 1–2):

  - Hardware selection brief: Pick hardware for a nightstand and justify choices (drawer slides, pulls, base glides).

  - Tabletop attachment demo: Install tabletop fasteners and explain how they allow seasonal movement.

  - Adjustment check: Install a concealed hinge and produce even door gaps using hinge adjustments.

  - Base fitting comparison: Test felt pads vs. leveling feet vs. casters and recommend for different floor types.